| Project | CERN-TMS |
| --- | --- |
| Date | 2006-10-25 |
| Reference | Cern-tms/pupe-api |
| Version | 0.6t2 Preliminary |
| Author | Dr Terry Barnaby |

Table of Contents

1.  References	1

2.  Introduction	1

3.  Overview	1

4.  Typical Data Required	3

5.  Control and Status Registers	3

6.  Synchronisation and Timing	8

7.  Interrupts	11

8.  Data Access	12

9.  Software Usage	13

10.  Notes	13

# References

- IT-3384/AB: Technical Specification for a new trajectory measurement system for the CERN Proton Synchrotron.

- Alpha Data’s TMS Tender “pre-design-1.5”.

- Emailed questions answered by Jeroen Belleman of CERN.

- Visit to CERN on 2006-06-20

# Introduction

This PUPE API document concerns the API between the PUPE FPGA firmware and the software running on the Module controllers.

# Overview

The TMS operates in cycles normally lasting 1.2 seconds but can be up to 2 seconds. Machine cycles can occur consecutively with 30ms between each although this gap can be over 1 second. The start of a cycle is initiated by the CYCLE_START timing signal and is terminated by the CYCLE_STOP timing signal. A cycle number counter is incremented on the end of a cycle. The cycle number counter can be set in software prior to the CYCLE_START event. A C-timer counter is zeroed on the start of cycle and is incremented every 1ms for timing purposes. The 10MHz System clock is used as a reference for this and also for synchronising the incoming timing signals.

During a Cycle, a set of particle bunches will be orbiting within the machine at a rate of about 477KHz (could be as low as 177KHz). There could be from 0 to 25 particle bunches orbiting at any one time. The integrals of the Sigma, DeltaX and DeltaY values for each bunch are stored in memory per orbit (turn). The memory is arranged as a simple two dimensional array, the first dimension being the bunch number within a bunch set  and the second being the orbit number.

A Cycle Timing Table has an entry every millisecond and stores the address in memory for the first bunch in the particle bunch set that occurred just after that time. The accuracy of bunch set timing is thus about 2us assuming an orbit rate of 477KHz.

A synchronisation system provides timing signals for the capture, DSP and memory storage system. This system synchronises itself to the bunches orbiting in the machine. As there are no bunches orbiting to start with, an external timing signal, FREF provides a reference to synchronise to prior to having a particle beam to lock on to. The FREF signal will be at the orbit rate.  It will need to be effectively delayed to match the position of the particular Pick Up in the ring. A register PLL_PHASE_DELAY defines this delay. The phase delay clock will be driven ant the sampling rate of 125MHz. Once the particle beam is present then the beam’s Sigma signal is used for accurate synchronisation, this will be at h x the orbit rate where h is the current harmonic number of the machine. The harmonic number of the machine defines how many RF Buckets there are and hence how many data sample sets are written to memory every orbit. The PLL system will produce a h x FREF signal synchronised to the incoming FREF signal or the Sigma signal. A well as the h x FREF signal the PLL will produce a Gate, BLR and other timing signals. The FREF signal will continue to provide timing for the orbit of the particle bunch set defining which of the RF Buckets is the first of the bunch set.

The PLL has a Phase Table that can be loaded on demand at any time but synchronised to FREF ie the start of a particle bunch (should this be the end to allow time for the change ?). The PLL’s initial frequency can also be configured.

During a cycle a number of events can occur which are signalled by timing signals. The injection signal indicates the point of injection of a BEAM into the machine. The calibration start and stop signals indicates a calibration period. The H-CHANGE signal indicates a harmonic change, ie the number of bunches in a bunch set changes. The timing and memory address at which these events happen are stored in a table for information. The PLL tables will need changing on H-CHANGE and other events.

Each PUPE has test and diagnostics functions. This enables the timing inputs to be driven manually. A set of data from various parts of the system can be sampled and stored into a memory block for diagnostics.

Data from previous cycles can be accessed from the memory while the system is processing the next cycle of data. In order to access a particular pattern of data from the results there may be a Data Pattern Engine to retrieve particular data patterns from the result data using DMA.

Each PUPE implements 3 Pick-Up processing channels. Each of these runs predominantly independent from each other.

# Typical Data Required

During a process cycle of 1 second the system will acquire about 40MBytes of data per Pick-Up channel. Not all of this data is required. Generally some pattern of data is required from the system dependent of the measurements being made. The typical measurements required include:

- Trajectory measurements of a single selected bunch through all Pick-Up’s over a small number (up to a few tens) of orbits. This would be best accomplished by having an FPGA on-board Data Pattern Engine to DMA the values for one of the bunches.

- The instantaneous position of a selected bunch through two selected pick-ups over a large number of orbits (~100k), hereafter referred to as a `phase space plot', or PSP. This would be best accomplished by having an FPGA on-board Data Pattern Engine to DMA the values for one of the bunches.

- The mean for each Pick-Up over a few hundred orbits. For a small number of selected pick-ups, this measurement may be requested at 1ms intervals in order to form a plot of the evolution of the mean position in these pick-ups over an acceleration cycle. This may be best achieved by calculating the mean on the fly and storing this in the Cycle Timing table. Alternatively an on-board Data Pattern Engine could calculate the mean values and DMA the results.

- The (weighted) mean of a selected bunch over all pick-ups over a few hundred orbits, hereafter referred to as the `Mean Radial Position', or MRP. This measurement may be requested at 1ms intervals to form a plot of the evolution of the MRP over an acceleration cycle. This may be best achieved by calculating the mean of a specified bunch on the fly and storing this in the Cycle Timing table. Alternatively an on-board Data Pattern Engine could calculate the mean values and DMA the results.

# Control and Status Registers

The control and status registers are mapped into memory. Each register is 32bits wide and is situated on a 64bit address boundary.

| Address | Name | Description |
| --- | --- | --- |
| 0x000000 | IMEM_REG | Internal block ram register (bank index) |
| 0x000008 | LOCKED | DCM lock status |
| 0x000010 | ADDR_REG | Memory page address register |
| 0x000018 | MEM_REG | Memory access register (bank, internal/external) |
| 0x000020 | IER | Interrupt enable register |
| 0x000028 | ISR | Interrupt status register |
| 0x000800 | FIRMWARE | Firmware magic and version. ‘C’ + ‘N’ + VersionMajor + VersionMinor. |
| 0x000808 | CONTROL_0 | Reset, Start, Stop, LoadPhaseTable, ,TimingMaster,TimingTerminate,Running,Error (RW) |
| 0x000810 | CYCLE_0 | The current cycle number. On write will be used as the cycle number for the next cycle on CYCLE_START. (RW) |
| 0x000818 | TIME_0 | The current cycle time in ms (RO) (could be in us) |
| 0x000820 | NUM_BUCKETS | The number of RF buckets (bunches) to be processed. Will be updated to the value written here on CYCLE_START and H_Change. (Maybe this should be a list of NUM_BUCKET values or they should be in the Phase tables ??) (RW) |
| 0x000828 | PLL_FREQUENCY | The initial value of the orbit frequency in Hz (RW) |
| 0x000830 | PLL_FREQUENCY_DELAY | The delay in ms (or orbits ?) after CYCLE_START at which the PLL frequency register is loaded with ORBIT_FREQUENCY. (This could be done with a bit in the Phase Table ??)(RW) |
| 0x000838 | PLL_PHASE_DELAY | PLL Phase delay setting. This is based on the position in the ring the PU is located and is used to effectively phase delay the FREF source to produce the signal FRefDelayed. (What values perhaps in master ADC clock @ 125MHz cycles ?) (RW) |
| 0x000840 | PLL_GAIN | Sets the gain of the PLL (what values ?) (RW) |
| 0x000848 | DPE_CONTROL | Data pattern engine control ??? |
| 0x000850 | TEST_0 | Sets test parameters (RW) |
| 0x000858 | TEST_1 | Sets diagnostics output (undefined at the moment)(RW) |
|  | PHASE_TABLE | Number indicating phase table in use (Phase table Counter). |
| 0x000860- | Pick Up Channel 1 registers. Same as for Pick Up channel 0 |  |
| 0x0008C0- | Pick Up Channel 2 registers. Same as for Pick Up channel 0 |  |
| 0x200000:0x3FFFFF | MEMORY | Memory window. 64 bits wide, suitable for slave mode DMA access |

Possible Extra Registers:

| Address | Name | Description |
| --- | --- | --- |
|  | PHASE_TABLE_NUMBER | Number indicating phase table in use (Phase table Counter). |
|  | ORBIT_NUMBER | The current orbit number |

The system memory is organised as follows:

| Bank | MEM_REG | IMEG_REG | Address | Description |
| --- | --- | --- | --- | --- |
| DDR 0 | 0x08 | 0xXX | 0x00000000 | Channel 0 data |
| DDR 1 | 0x09 | 0xXX | 0x00000000 | Channel 1 data |
| DDR 2 | 0x0A | 0xXX | 0x00000000 | Channel 2 data |
| DDR 3 | 0x0B | 0xXX | 0x00000000 | Test data |
| BlockRAM 0 | 0x00 | 0x00 | 0x200000 | Cycle Information table channel 0 |
| BlockRAM 0 | 0x00 | 0x00 | 0x200200 | Timing Phase Table channel 0. |
| BlockRAM 1 | 0x00 | 0x01 | 0x200000 | Cycle Timing table channel 0 (8KBytes) |
| BlockRAM 2 | 0x00 | 0x02 | 0x200000 | Pattern Read Buffer channel 0 |
| BlockRAM 4 | 0x00 | 0x04 | 0x200000 | Cycle Information table channel 1 |
| BlockRAM 4 | 0x00 | 0x04 | 0x200200 | Timing Phase Table channel 1. |
| BlockRAM 5 | 0x00 | 0x05 | 0x200000 | Cycle Timing table channel 1(8KBytes) |
| BlockRAM 6 | 0x00 | 0x06 | 0x200000 | Pattern Read Buffer channel 1 |
| BlockRAM 8 | 0x00 | 0x08 | 0x200000 | Cycle Information table channel 2 |
| BlockRAM 8 | 0x00 | 0x08 | 0x200200 | Timing Phase Table channel 2. |
| BlockRAM 9 | 0x00 | 0x09 | 0x200000 | Cycle Timing table channel 2 (8KBytes) |
| BlockRAM A | 0x00 | 0x0A | 0x200000 | Pattern Read Buffer channel 2 |

Possible Extra Memory Areas:

| Bank | MEM_REG | IMEG_REG | Address | Description |
| --- | --- | --- | --- | --- |
| BlockRAM |  |  |  | Mean of all bunches Sigma values. One entry per millisecond. |
| BlockRAM |  |  |  | Mean of Sigma values for a particular bunch. The bunch number can be selected in a register. One entry per millisecond. |

The Register bit definitions are:

| Name | Bits | Description |
| --- | --- | --- |
| Control.Reset | 0 | Resets the Pick Up channel |
| Control.Start | 1 | Starts the next processing cycle. The C-Timer counter will be reset to 0 and the PhaseTable counter will also be set to 0. The actual start will happen on the next CYCLE_START timing signal. |
| Control.Stop | 2 | Manual stop cycle. Normally the CYCLE_STOP timing signal will terminate the cycle |
| Control.LoadPhaseTable | 3 | Resets the Phase Table counter back to 0. |
| Control.TimingMaster | 5 | Sets the logic to connect the digital timing signals to the timing bus. Used on the card connected to the digital timing signal input panel. |
| Control.TimingTerminate | 6 | Sets the logic to terminate the timing bus. Used in the last card on the timing bus. |
| Control.Running | 16 | System is running (RO) |
| Control.Error | 17 | System is in Error (RO) |
| Test.TestOutput | 0-3 | Selects the signal which will be output on the channels digital output connector. 0 – none, 1 – generated FREF. |
| Test.DataSource | 4 | Selects input data source from DDR3 bank rather than from ADC’s for testing |
| Test.ClockSource | 5 | Selects internal 10MHz clock rather than digital input system clock |
| Test.FRefSource | 6 | Selects internal Fref source (477 Khz ?) |
| Test.CycleStart | 16 | Timing bus signal |
| Test.CycleStop | 17 | Timing bus signal |
| Test.CalStart | 18 | Timing bus signal |
| Test.CalStop | 19 | Timing bus signal |
| Test.Injection | 20 | Timing bus signal |
| Test.HChange | 21 | Timing bus signal |
| Test.TimingDrive | 23-31 | Sets the board to drive the systems timing bus with the values from this register. This is a bit mask allowing each timing signal to be driven independently. |

The table sizes are as follows:

| Table | Size | Time |
| --- | --- | --- |
| Cycle Data table | 256 MBytes actual used size rounded down to be a multiple of 6 bytes the data entry size | 4.46 seconds 21 bunches |
| Cycle timing table | 17840 Bytes. Each entry is 8 bytes. Does this need to be bigger to allow the system to be set to capture single bunches with more past data held in the Cycle Data table ? | 4.46 seconds 21 bunches |
| Cycle Information table | 512 Bytes for 4 Cycles of information |  |
| Timing Phase Table | 16 * 512 Bytes |  |

# Synchronisation and Timing

One of the major aspects of the PUPE is its synchronisation system. The synchronisation system produces internal timing signals for the PUPE based on external timing signals and the analogue Sigma data source. Each Pick-Up Channel has the following input timing signals:

| Name | Description |
| --- | --- |
| SYSTEM_CLOCK | Master 10MHz system clock. The ADC’s 125 Mhz sampling clock will be synchronised to this clock and all of the digital timing signals, except the Injection signal, will re-synchronised to this clock within each FPGA |
| FREF | Reference frequency. (Up to 477KHz) |
| CYCLE_START | Start of a machine cycle |
| CYCLE_STOP | End of Last Flat Top, effectively end of cycle |
| CAL_START | Start of calibration period |
| CAL_STOP | End of calibration period |
| INJECTION | Injection |
| H_CHANGE | Harmonic changes |
| SPARE | Spare input not used |
| SIGMA | The amplitude of the signal from the pick up. This is an analogue signal |

There is a set of Phase Tables to provide PLL frequencies and to control the synchronisation process. There would be 16 or more Phase tables. Each would have 512 x 8bit entries. Thus the memory requirement would be 16 x 512 bytes.

On each timing signal event the FPGA would switch to the next table in the 16 table array by simply incrementing a high bits address counter. Note that instead of this simple method of Phase table change, we could have a separate timing signal selection switch table to choose which signal would effect a Phase Table change or even include this switch information in the main Phase/Switch tables by making them wider than 8 bits.

Normally the set of Phase Tables would be loaded by software before starting a processing cycle. However it would also be possible to load them during a processing run making sure that the current Phase Table in use is not altered. The software would load the table via the shared memory interface.

The following shows a possible configuration of the Phase table, the actual parameters and positions are yet to be defined:

| Name | Bit | Description |
| --- | --- | --- |
| FRefSel | 0 | Selects the FRefDelayed timing input or the ADC Sigma as the frequency reference to lock on-to. This changes the both the input reference source, and which of PllFRef or PllFRefHarm to use as the feed back signal in the PLL. |
| PllFRef | 1 | Phase table to synchronise to FRefDelayed. One cycle of FRef |
| PllFRefHarm | 2 | Phase table to synchronise to Harmonic number * FREF ie Sigma. Harmonic number of cycles. |
| Gate | 3 | Gate signal for data integration |
| Blr | 4 | Base line restore signal |
| Acquire | 5 | Acquire data signal. This specifies the ADC samples to integrate and is used as the bunch clock. |
| Spare0 | 6 |  |
| Spare1 | 7 |  |

The following diagram gives an indication of the possible FPGA Firmware blocks to implement the synchronisation system to match the above Phase Table layout.

# Interrupts

Interrupts can be generated on each of the following events:

| Event | Interrupt Bit | Description |
| --- | --- | --- |
| CycleStart | 0 | A processing cycle has started |
| CycleStop | 1 | A processing cycle has completed |
| Error | 2 | An error has occured |
| FREF | 16 |  |
| CYCLE_START | 17 | Timing input |
| CYCLE_STOP | 18 | Timing input |
| CAL_START | 19 | Timing input |
| CAL_END | 20 | Timing input |
| INJECTION | 21 | Timing input |
| H_CHANGE | 22 | Timing input |
| SPARE | 23 | Timing input |

# Data Access

Access to the data from the PUPE is provided through a shared memory interface. To reduce the shared memory access window a simple 2MByte paged system is used to access the FPGA’s block RAM and on-board DDR. The on-board DDR is organised as four banks of 256 Mbytes. Within this memory is stored the main data tables for each of the Pick-Up channels.

Each of the Pick-Up channels has its own data table stored within its own bank of 256MBytes of DDR. The format of this data table is a circular buffer of Sigma, DeltaX, DeltaY and Spare 16bit signed values in little endian order. The Spare entry is there to give 64bit data alignment for efficiency and simplicity of access, it may be used to contain other data in the future (microsecond timer ?). For each particle bunches orbit there will be h sets of data stored where h is the machines harmonic number. The machines harmonic number can change on the H-CHANGE timing event and hence the number of sets of data stored per orbit will change. The points of change are recorded in the Cycle information table. The data table can contain many cycles of many orbits of data.

| 0 | 2 | 4 | 6 | 8 | 10 | 12 |
| --- | --- | --- | --- | --- | --- | --- |
| Sigma | DeltaX | DeltaY | Spare | Sigma | DeltaX | ... |

In the Block RAM of the FPGA memory there is a Cycle Timing table. This circular buffer consists of one timing entry per millisecond. Each entry in the timing table contains a 32bit unsigned “Cycle Number” field and a  32bit unsigned memory address. An entry is made in the timing table every millisecond. The memory address points to offset in the data table which contains data for that time. This will be the address of the first bunch in the next orbit after this time.

| 0 | 4 | 8 | 16 | 20 |
| --- | --- | --- | --- | --- |
| Cycle Number | Data Address | Cycle Number | Data Address | .... |

There is a Cycle Information table, contained within internal BLOCKRAM memory which contains timing positions for particular events. There will be information for up to 4 machine cycles. An example Cycle information table follows.

| Address | Type | Millisecond Time | Orbit Number |
| --- | --- | --- | --- |
| 0 | CycleStart, Nbuckets, CycleNumber | 0 | 0 |
| 4 | CalStart | 100 | 0 |
| 8 | CalStop | 150 | 10 |
| 12 | Injection | 200 | 2 |
| 16 | HChange, NBuckets | 300 | 100 |
| 20 | HChange, NBuckets | 400 | 250 |
| 24 | HChange, NBuckets | 450 | 0 |
| 28 | CycleStop | 800 | 33 |

# Software Usage

There follows a typical TMS Cycle measurement scenario from the controlling software’s perspective.

- Software configures the Pick Up engines main registers including the CONTROL, PLL_FREQUENCY_DELAY, PLL_PHASE_DELAY, PLL_GAIN and TEST registers.

- The Software starts of the next processing cycle. It loads the set of Phase Tables for the cycle. It sets the PLL_FREQUENCY and CYCLE number register and initiates a processing CYCLE by setting the START bit in the Control register.

- The PUPE Firmware sets the Phase table counter to 0 on the CYCLE_START timing signal and starts processing the incoming data. On timing input events the PUPE Firmware increments the Phase Table counter to the next phase table. For example on H-CHANGE events the system will change to use the next Phase table.

- The PUPE Firmware will record each timing event into the Cycle information table together with the millisecond time and orbit number when it occurred.

- On the CYCLE_STOP timing input the processing will stop and the software will be informed by interrupt. The software can now access the resulting data possibly through a Data Pattern Engine implemented in the FPGA or by direct memory accesses.

- The next processing cycle can be started immediately after the previous cycle even while the previous cycles data is being collected.

# Notes

- The C-Timer could be implemented as a microsecond counter. This would give the ability for more accurate timing for future developments.

- Could store microsecond time in the data table. This would increase memory requirements but would give more accurate timing information.

- We might want the ability for the FPGA to DMA a selected pattern of data to the host. For example select the data for bunch 1 and copy that data to the host. This would be to improve the performance of the data retrieval.

- One thing we will have to consider is how fast we need to get at the data in memory. CERN will want to be able to take out a selection of data before the next machine cycle. With the paging registers, the table indirection (Cycle Timing to Data) and the fact that they may only want 6bytes here and there over a PCI bus this may be slow. We thus may need to add a data gathering facility in the FPGA itself such as a function that would DMA every 21st set of 6byte samples stating from a given address to the host.

- Another option on the data, as we have more memory that the original spec, is to store data timing information with the data itself rather than have a separate table. We could packetise the data with a simple header containing the channel number and time followed by 21 sets of Sigma/DeltaX/DeltaY data. This would make finding a particular time a bit harder/slower but we could store more accurate times on the data.

- It might be best to calculate and store some of the data that requests for data would require on the fly. For example it would be relatively easy to calculate the mean of DeltaX for bunch 1 over a 1ms period on the fly.

- The Cycle Timing table might want to be bigger to allow the system to be set to capture single bunch data over a longer time period.